Experiment 7

Pulse Shaping and the Nyquist Criterion

Arkaprava Dutta

B.Tech, Electronics and Computer Engineering

Cooch Behar Government Engineering College  |  Digital Communication Laboratory

# 1. Aim and simulation parameters

To compare rectangular, sinc, raised-cosine (RC) and root-raised-cosine (RRC) pulses, and to study the bandwidth–timing trade-off governed by the roll-off factor.

| Parameter | Value |
| --- | --- |
| Symbol period T | 1.0 s |
| Sampling rate | 16 samples/symbol |
| Filter span | ±5T (10 symbols) |
| Modulation | BPSK, symbols ±1 |
| Roll-off factors tested | α = 0, 0.25, 0.5, 1.0 |
| Implementation | Python 3 / NumPy — filters coded from closed-form definitions |

# 2. Theory used in the implementation

## 2.1 Nyquist’s first criterion

A pulse produces zero intersymbol interference if its impulse response satisfies h(kT) = 1 for k = 0 and h(kT) = 0 for all k ≠ 0. Equivalently, its folded spectrum must be flat:

Σₙ H( f − n/T ) = constant

## 2.2 Raised cosine

hʀᴄ(t) = sinc(t/T) · cos(παt/T) / [ 1 − (2αt/T)² ]

The expression is 0/0 at two points. Both limits were evaluated by L’Hôpital’s rule and hard-coded rather than left to floating-point arithmetic:

| Singular point | Analytic limit |
| --- | --- |
| t = 0 | 1 |
| t = ±T/2α | (π/4) · sinc(1/2α) |

## 2.3 Root raised cosine

hʀʀᴄ(t) = [ sin(π(t/T)(1−α)) + 4α(t/T)cos(π(t/T)(1+α)) ] / [ π(t/T)(1 − (4αt/T)²) ]

| Singular point | Analytic limit |
| --- | --- |
| t = 0 | 1 + α(4/π − 1) |
| t = ±T/4α | (α/√2)[(1 + 2/π)sin(π/4α) + (1 − 2/π)cos(π/4α)] |

Since H_RRC( f ) = √H_RC( f ), the cascade of a transmit RRC and a matched receive RRC is exactly a raised cosine. Only the pair is Nyquist — never the individual filter. Because α = 0 collapses both expressions to sinc(t/T), the code guards the 1/2α and 1/4α singularity locations with an α > 0 test instead of letting them evaluate to infinity.

# 3. Predictions recorded before simulation

As required, the expected physical effect of each parameter variation was written down before any code was run.

| Variation | Predicted physical effect |
| --- | --- |
| Rectangular pulse | Perfectly compact in time, but the spectrum is a sinc with a −13 dB first sidelobe and only 1/f decay, giving unacceptable adjacent-channel leakage. |
| α = 0 (ideal sinc) | Minimum possible bandwidth 1/2T, but tails decay only as 1/t, so extreme sensitivity to timing error and very slow settling. Truncation should hurt this case most. |
| α increasing 0 → 1 | Bandwidth grows as (1+α)/2T; time tails decay faster (1/t³); peak overshoot and PAPR fall; the eye should open wider horizontally. |
| RC sampled at kT | Exactly 1 at k = 0 and exactly 0 elsewhere, for every α. |
| RRC alone at kT | Not zero at k ≠ 0 — RRC is not a Nyquist pulse by itself. |
| RRC ⊗ RRC cascade | Should reproduce the RC sample values and restore the zero crossings. |

# 4. Implementation tasks

| Task | Description | Where |
| --- | --- | --- |
| 23 | Upsampled symbol impulse train | Figure 3, panel 1 |
| 24 | Raised-cosine roll-offs α = 0, 0.25, 0.5, 1 | Figures 1, 2, 4 |
| 25 | Impulse and frequency responses | Figures 1 and 2 |
| 26 | Verify zero crossings at symbol intervals | Validation 1, Figure 3 panel 2 |
| 27 | Cascade transmitter and receiver RRC filters | Validation 2, Figure 3 panel 3 |

# 5. Results

## 5.1 Pulse responses

Figure 1 — Rectangular, sinc, raised-cosine and root-raised-cosine impulse responses.

All RC pulses pass through unity at t = 0 and cross zero at every other integer multiple of T. Increasing α visibly compresses the tails: at α = 1 the ringing is essentially gone beyond ±2T, whereas at α = 0 it is still significant at ±5T. The RRC pulses do not cross zero at kT — their peak is 1 + α(4/π − 1) > 1 — exactly as predicted.

## 5.2 Frequency responses

Figure 2 — Magnitude spectra in dB. Dashed vertical lines mark the Nyquist edge 1/2T.

RC responses are flat in the passband, odd-symmetric about f = 1/2T (the −6 dB point), and strictly band-limited to (1+α)/2T. The rectangular pulse never truly ends: its sidelobes fall only about 20 dB per decade. RRC responses share the same band edges but have a square-root-shaped roll-off, so their skirts are less steep in dB than the corresponding RC.

## 5.3 Pulse-shaped stream

Figure 3 — Impulse train (Task 23), RC-shaped stream (Task 26) and cascaded RRC output (Task 27).

The RC-shaped waveform passes exactly through the ±1 markers at every symbol instant for both α = 0.25 and α = 1, while the α = 0.25 waveform overshoots noticeably more between symbols. The transmit-RRC → receive-RRC cascade output again lands precisely on the markers, confirming that the matched pair restores the Nyquist property.

## 5.4 Bandwidth versus roll-off

Figure 4 — Absolute bandwidth and excess bandwidth as functions of roll-off.

## 5.5 Eye diagrams — four-way comparison

Figure 5 — Eye diagrams over 400 random symbols. The red line marks the ideal sampling instant.

The rectangular pulse gives a perfectly rectangular eye — maximally open at every instant, since it has no tails at all. That is precisely why it is the time-domain optimum and the frequency-domain disaster. The sinc eye is open at t = 0 but its crossings smear badly on either side. RC α = 0.25 is visibly cleaner, and RC α = 1 produces a wide, well-defined eye with tightly bunched zero crossings.

## 5.6 The bandwidth–timing trade-off quantified

Figure 6 — Worst-case eye opening versus timing offset, and jitter margin before eye closure.

# 6. Mandatory validation

## 6.1 Validation 1 — RC response sampled at integer symbol intervals

This is the validation specified in the assignment: the overall raised-cosine response sampled at t = kT and tabulated.

| k | α = 0 | α = 0.25 | α = 0.5 | α = 1.0 |
| --- | --- | --- | --- | --- |
| 0 | 1.000 | 1.000 | 1.000 | 1.000 |
| ±1 | 3.90e−17 | 3.68e−17 | 3.06e−17 | 1.30e−17 |
| ±2 | 3.90e−17 | 3.06e−17 | 1.30e−17 | 2.60e−18 |
| ±3 | 3.90e−17 | 2.21e−17 | 8.95e−34 | 1.11e−18 |
| ±4 | 3.90e−17 | 1.30e−17 | 2.60e−18 | 6.19e−19 |
| ±5 | 3.90e−17 | 5.25e−18 | 4.97e−34 | 3.94e−19 |

The largest off-centre value anywhere is 3.9 × 10⁻¹⁷ — machine epsilon. The Nyquist criterion is satisfied to the limit of double precision for every roll-off tested.

## 6.2 Validation 3 — ISI budget and peak-to-average power

| α | peak ISI | Σ¦ISI¦ | eye opening | PAPR (dB) |
| --- | --- | --- | --- | --- |
| 0 | 3.9e−17 | 3.9e−16 | 1.000000 | 5.39 |
| 0.25 | 3.7e−17 | 2.2e−16 | 1.000000 | 4.61 |
| 0.5 | 3.1e−17 | 9.2e−17 | 1.000000 | 3.59 |
| 1.0 | 1.3e−17 | 3.5e−17 | 1.000000 | 1.53 |

PAPR falls by about 3.9 dB from α = 0 to α = 1 — the quantitative form of the bandwidth-versus-linearity trade-off. Note that the eye-opening column reads 1.000000 for every α: measured at the ideal instant alone, this metric cannot discriminate between roll-offs at all. Section 6.4 addresses that.

## 6.3 Validation 4 — bandwidth

| α | theory (1+α)/2T | 99 % energy BW | −3 dB BW | excess BW |
| --- | --- | --- | --- | --- |
| 0 | 0.500 | 0.498 | 0.471 | 0 % |
| 0.25 | 0.625 | 0.512 | 0.467 | 25 % |
| 0.5 | 0.750 | 0.557 | 0.432 | 50 % |
| 1.0 | 1.000 | 0.678 | 0.365 | 100 % |

## 6.4 Validation 7 — bandwidth versus timing margin

Worst-case eye opening was computed as |h(Δt)| − Σ|h(kT + Δt)| over k ≠ 0, that is, the ISI margin under the worst possible data pattern, by evaluating the closed-form pulse directly at the offset instants.

| α | BW (1+α)/2T | eye at Δt = 0 | eye at 5 % T | eye at 10 % T | eye closes at |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.500 | 1.0000 | 0.7682 | 0.5321 | 0.220 T |
| 0.25 | 0.625 | 1.0000 | 0.8327 | 0.6591 | 0.290 T |
| 0.5 | 0.750 | 1.0000 | 0.8981 | 0.7868 | 0.400 T |
| 1.0 | 1.000 | 1.0000 | 0.9552 | 0.8967 | > 0.5 T |

This is the core result of the experiment. At perfect timing all four roll-offs are identical (1.0000) — the Nyquist criterion alone cannot discriminate between them. The trade-off only becomes visible once timing error is introduced: doubling the bandwidth from 0.5/T to 1.0/T more than doubles the jitter tolerance, from 0.22 T to beyond 0.5 T. A 10 % timing error costs the ideal sinc 47 % of its eye but costs α = 1 only 10 %.

## 6.5 Validation 8 — four-way pulse comparison

| Pulse | max ¦h(kT)¦, k ≠ 0 | Tail decay | Null-to-null BW | 1st sidelobe |
| --- | --- | --- | --- | --- |
| Rectangular | 0 | 1/f in frequency | infinite | −13.2 dB |
| Sinc (α = 0) | 3.9e−17 | 1/t | 0.500/T | −40.3 dB |
| RC α = 0.25 | 3.7e−17 | 1/t³ | 0.625/T | −57.0 dB |
| RC α = 1.0 | 1.3e−17 | 1/t³ | 1.000/T | −68.1 dB |
| RRC α = 0.25 | 6.4e−02 | 1/t³ | 0.625/T | −48.5 dB |

The RRC row is the one that fails the zero-ISI test — by fourteen orders of magnitude, not by rounding. That single number is the entire justification for splitting the shaping between transmitter and receiver.

# 7. Discrepancies and the tests used to diagnose them

## 7.1 The cascaded RRC pair does not match RC exactly

Validation 2 gave a worst-case deviation of 8.3 × 10⁻² at α = 0 and 6.2 × 10⁻³ at α = 0.25, against roughly 10⁻¹⁷ for the analytic RC.

Diagnostic test: re-run the cascade with filter spans of 10, 20, 40 and 80 symbols.

| Filter span | α = 0 | α = 0.25 | α = 0.5 | α = 1.0 |
| --- | --- | --- | --- | --- |
| 10 T | 8.31e−2 | 6.25e−3 | 4.69e−3 | 1.49e−4 |
| 20 T | 1.42e−2 | 7.87e−5 | 1.74e−5 | 5.62e−6 |
| 40 T | 5.86e−3 | 5.80e−6 | 1.46e−6 | 4.00e−7 |
| 80 T | 2.71e−3 | 6.14e−7 | 1.54e−7 | 4.01e−8 |

The error falls monotonically with span, and fastest for large α. Conclusion: this is window truncation, not a formula error. Truncating the RRC at ±5T is a rectangular windowing in time, and the exact square-root factorisation H_RRC² = H_RC holds only for the untruncated filter. α = 0 converges slowest precisely because its tails decay as 1/t, so the discarded energy is largest — a result consistent with theory rather than contradicting it. This is why real modems use α ≥ 0.2 and spans of 8–12 symbols, and why the ideal sinc is never implemented.

## 7.2 Measured −3 dB bandwidth decreases as α increases

This is the opposite of the expected trend.

Diagnostic test: inspect where each RC curve crosses the relevant threshold in Figure 2. Every RC response passes through exactly −6 dB (amplitude 0.5) at f = 1/2T regardless of α, so the −3 dB point sits at the start of the roll-off skirt, which moves inward as (1−α)/2T.

Conclusion: the metric was mis-chosen, not the filter. The −3 dB point measures the flat-passband edge, not the occupied bandwidth. For Nyquist pulses the correct measures are the null-to-null or 99 % energy bandwidth, both of which track (1+α)/2T as predicted.

## 7.3 The 99 % energy bandwidth sits below the theoretical value

At α = 1 the measured value is 0.678 against a theoretical 1.000. This is not an error either: (1+α)/2T is the absolute, null-to-null bandwidth, whereas the last 1 % of the RC skirt energy lies in the outer part of the roll-off region. The two definitions coincide only for a brick-wall spectrum, which is exactly what the α = 0 row shows (0.498 against 0.500).

## 7.4 Singularity check

Each analytic limit was compared against the filter evaluated 10⁻⁷ away from the singular point. All agreed to six or seven decimal places — for example, the RRC at α = 1, t = T/4 gives an analytic 1.0000000000 against a numerical 0.9999997998. Without the explicit limits those samples evaluate to 0/0 = NaN and the convolution output collapses entirely.

# 8. Conclusions

- Raised-cosine pulses satisfy the Nyquist zero-ISI criterion exactly for all roll-offs, verified to 10⁻¹⁷ at every sampled symbol instant.

- RRC alone is not a Nyquist pulse; only the matched transmit/receive pair is. This is the whole reason the split is used — it delivers matched filtering (optimum SNR in AWGN) and zero ISI simultaneously.

- The roll-off factor is a direct trade: bandwidth grows as (1+α)/2T while tail decay improves from 1/t to 1/t³, PAPR drops about 3.9 dB, and jitter tolerance rises from 0.22 T to beyond 0.5 T.

- The trade-off is only measurable under timing error. At Δt = 0 every roll-off gives an identical unit eye, so a validation performed only at the ideal sampling instant has not actually demonstrated the effect the experiment is about.

- Rectangular pulses are time-compact but spectrally unacceptable; the ideal sinc is spectrally optimal but unrealisable and fragile under truncation and timing error. Both extremes are ruled out by the same trade-off, approached from opposite ends.

- Practical systems therefore choose α = 0.2 to 0.35 — DVB-S2 uses 0.20 to 0.35, and LTE-style SC-FDMA about 0.22.

# 9. Files accompanying this report

| File | Contents |
| --- | --- |
| exp7_pulse_shaping.py | Full implementation, tasks 23–27 |
| fig1_pulse_responses.png | Rectangular, sinc, RC and RRC impulse responses |
| fig2_frequency_responses.png | Magnitude spectra in dB |
| fig3_pulse_shaped_stream.png | Impulse train, RC-shaped stream, RRC cascade output |
| fig4_bandwidth_vs_rolloff.png | Bandwidth and excess bandwidth versus α |
| fig5_eye_diagrams.png | Eye diagrams for all four pulse types |
| fig6_timing_sensitivity.png | Eye opening versus timing offset; jitter margin |
| validation_tables.txt | All eight validation tables, raw numerical output |